Website: www.fh-rfid.com.cn

UHF Laundry Tag

UR 3515-S

Model

UR 3515-S

RFID Standard	EPC Class 1 Gen 2,1S018000-6C

Frequency

865~868MHz or 902~928MHz

Length * Width

35*15 mm(2.7*0.6 in)

Thickness

1.5 mm(0.06 in)on chip location only,Rest of tag is 0.55 mm(0.02in)

Material

Textile

Weight	0.5g

Chip Type	M730/ NXP Ucode9

Memory

EPC 128-bit, User 0-bit / EPC 96-bit, User 0-bit

Data Storage

20 years

Re-write

100,000 times

Read Range(Fixed Reader)

0-5m(ERP=2W)

Read Range(Handheld Reader)

0-3m(Using 30dBm power)

Warranty

200 washing cycles or 3 years, whichever comes first

Installation

Sew in the rolled edge or in the woven label sleeve (cannot be sewn onto metal wires or chip modules during sewing)

Washing Method	Laundry, Dry cleaning

Water Extraction pressure

60 bars

Washing Temperature

90°C(194°F)/15 minutes

Chemical Resistance	Can withstand all chemicals in the normal industrial washing

Pre-drying in Tumble	180°C(356°F)/30 minutes

lroning Tempereature

180°C(356°F)/10 seconds

Humidity/Temperature-Operating

-20~+85°C(-4~+185°F)/8~95%RH

Sterilization Tempereature	135°C(275°F)/20 minutes

Personalization	Serial number,LOGO printing, Encoding,Antenna design.etc

Applications

Industrial washing/Management of uniforms Medical apparel management

Military clothing management

Uniform management

Application

Hotel linen management

Hospital linen management

Website: www.fh-rfid.com.cn

Linen rental management

Read Range