Website: www.fh-rfid.com.cn

UHF Laundry Tag

UR 7010-H

Model

UR 7010-H

RFID Standard	EPC Class 1 Gen 2 , ISO18000-6C

Frequency

845MHz ~ 950MHz(worldwide)

Thickness

1.5 mm(0.06 in)on chip location only,Rest of tag is 0.55 mm(0.02in)

Length * Width	70*10 mm(2.7*0.6 in)

Weight	0.6g

Material

Textile

Memory

EPC 128bits User0/EPC 96bits,User 32bits

Chip Type	M730/NXP Ucode7M

Data Storage	20 years

Re-write

100,000 times

Read Range(Handheld Reader)

0-7m(Using ATID AT880 handheld device)

Read Range(Fixed Reader)	0-9m(ERP=2W)

Installation

Heat-sealing:Heat-seal the tag onto the textile at 215°C and pressure greater than 0.6MPa for 15-20 seconds,or thread installation, or woven label sleeve installation

Warranty

200 washing cycles or 3 years, whichever comes first

Water Extraction pressure

60 bars

Washing Method	Laundry, Dry cleaning

Chemical Resistance	Can withstand all chemicals in the normal industrial washing

Washing Temperature

90°C(194°F)/15 minutes

lroning Tempereature

180°C(356°F)/10 seconds

Pre-drying in Tumble	180°C(356°F)/30 minutes

Sterilization Tempereature	135°C(275°F)/20 minutes

Humidity/Temperature-Operating

-20~+85°C(-4~+185°F)/8~95%RH

Applications

Industrial washing/Management of uniforms Medical apparel management

Military clothing management

Personalization	Serial number,LOGO printing, Encoding,Antenna design.etc

Uniform management

Application

Hotel linen management

Hospital linen management

Website: www.fh-rfid.com.cn

Linen rental management

Read Range